# Optical payload removal and refit with a connector discrepancy

ESV-S01-SOP-01   Payload satellite   Simulation procedure

Remove and refit the optical payload during an unpowered ground-integration rehearsal. Resolve a conflicting connector instruction before releasing the harness and mount, then prove that the final configuration matches the approved starting state.

Use this procedure to author and assess a controlled simulation with the Payload satellite model, version 5. This is a concept-model training exercise. It does not authorise real hardware work or establish flight qualification.

# Before the exercise

Load the saved version 5 scene. The scenario owner supplies the harness map, isolated-power evidence, ESD condition, supported and parked poses, removal path and assessment tolerances. Keep a separate fixture root. No real hardware is energised.

Identity and provenance: ESV-S01-SOP-01 · S01 · model v5 · 25 October 2026

Explore the model and related procedures

# 1 Establish the task configuration

Model labels: bus, base, optics

Match the spacecraft, fixture and optical payload to the saved version 5 starting configuration. Select the three correct objects and save the baseline pose.

Check: Proceed only when all three identities and the task revision agree.

# 2 Secure the ground support

Model labels: base, foot_rf, foot_lf, foot_rr, foot_lr

Confirm all four support locations and put the simulated work package into the secured-fixture state. Keep the fixture independent of the spacecraft.

Check: A missing or misidentified support prevents the removal sequence.

# 3 Establish the unpowered work state

Model labels: conn_ru, conn_rl, conn_lu, conn_ll

Identify all four connector positions and acknowledge the scenario's power-isolated and ESD-ready condition before disconnecting anything.

Check: Power state is supplied by the scenario or an instrument record; the mesh does not prove it.

# 4 Trace the intended harness

Model labels: conn_ru, cable_r, cable_r_sleeve

Follow the training-defined right harness from its connector to the optical payload. Compare the actual connector identity with the task card.

Check: This fictional exercise maps the harness to conn_ru; an actual programme supplies its own mapping.

# 5 Catch the planted discrepancy

Model labels: conn_ru, conn_rl, cable_r_head_clip

Present a task card calling for the lower connector while the approved training configuration calls for the upper one. Learner must place a hold, capture both identities and request the reviewed correction.

Check: Continuing with the lower connector is a critical error. The approved branch corrects the task card and resumes from this checkpoint.

# 6 Open the access area

Model labels: optic_hatch_r, optics, cable_r_head_clip

Select the right optical access cover, expose the cable retention area and identify the retaining clip. Use a declared detachable-cover animation if hinge travel has not been established.

Check: Do not infer an engineered hinge axis or open the opposite-side cover.

# 7 Release the harness in sequence

Model labels: cable_r_head_clip, cable_r_sleeve, cable_r, conn_ru

Release the simulated retention, disconnect the reviewed connector and move the right harness into the defined stowed position.

Check: Record the selected parts and sequence; prevent pulling the payload while the harness remains connected.

# 8 Support and remove the optical payload

Model labels: optics, S01-PMI, bus

Set the supported-payload hold point, release the payload mount interface and withdraw the optical assembly along the authored removal path into its parked pose.

Check: The mount cannot be released before the support acknowledgement. Fastener-level actions require separately addressable fasteners; do not invent them.

# 9 Refit to the correct interface

Model labels: S01-PMI, optics, bus

Return the optical assembly to its saved baseline pose and align its mount interface with the bus. Reject a reversed or incomplete seating state.

Check: Assess against declared simulation pose tolerances, not unsupported flight alignment claims.

# 10 Restore the harness and retention

Model labels: conn_ru, cable_r, cable_r_sleeve, cable_r_head_clip

Reconnect the correct connector, restore the harness route and sleeve position, and re-engage retention in the reviewed order.

Check: The lower connector remains a decoy. Missing retention or the wrong route prevents completion.

# 11 Close and verify clearance

Model labels: optic_hatch_r, solar_r, solar_l, objective

Restore the access cover. Check the harness/cover against both folded arrays and keep the objective in the no-contact zone.

Check: Use reviewed geometric envelopes where available; visual appearance alone is not a clearance measurement.

# 12 Demonstrate completion and hand over

Model labels: S01-PMI, optics, conn_ru, cable_r_head_clip, base

Compare final and baseline configuration, review the discrepancy/recovery record and submit the evidence package for the next engineer.

Check: Record identity, order, positions, hold resolution and required external test records. Do not infer torque, electrical continuity or powered performance from video alone.

# Assessment and completion

Score component identification 25, sequence 20, discrepancy handling 20, restored configuration 20 and evidence handover 15. Wrong-connector continuation, unsupported payload release or work before power-state confirmation prevents completion. Preserve both the original discrepancy and the accepted recovery.

# Authoring and headless handoff

Bind the optics assembly, its cover, harness and mount to their actual component IDs. Configure supported, disconnected, parked, seated and retained states. Use declared detachable-cover and removal transforms; do not infer hinge travel or fastener-level actions from the mesh. Instrument results remain explicit scenario evidence.

Reuse the exact GLB and component identifiers above. Verify the supported Genesis import and binding path; the current Creator scene driver can generate a different model. This document and its JSON describe authoring intent, not completed interactions. Preserve one model revision and record actual lesson, job and review identifiers during execution.