# Service module exchange with connector recovery

ESV-S05-SOP-01   Serviceable satellite   Simulation procedure

Rehearse exchanging the middle bay module while maintaining spacecraft support and preserving the intended harness configuration. Resolve a wrong-connector instruction before restoring the bay.

Use this procedure to author and assess a controlled simulation with the Serviceable satellite model, version 4. This is a concept-model training exercise. It does not authorise real hardware work or establish flight qualification.

# Before the exercise

Use the unpowered serviceable-satellite concept on its ground stand. Record the starting model pose and a training connector map approved by the exercise author. Supply module support, removal path, connector pairing and completion criteria as scenario data; the geometry alone does not establish them.

Identity and provenance: ESV-S05-SOP-01 · S05 · model v4 · 25 October 2026

Explore the model and related procedures

# 1 Identify the supported assembly

Model labels: stand, bus, bay.module2

Select the spacecraft, its independent stand and the middle service module. Match the model version and save the starting configuration.

Check: The stand remains fixed; a different module identity prevents progression.

# 2 Establish the work state

Model labels: bus.sideport, bay.harnessleft, bay.harnessright

Read the supplied isolation record and place the exercise in its unpowered maintenance state. Identify both harness routes before opening the bay.

Check: Require the recorded isolation state; appearance is insufficient evidence.

# 3 Release and park the door

Model labels: door, door.latchupper, door.latchmiddle, door.latchlower

Release the three labelled latch objects in the authored order and move the door to its defined access pose.

Check: All latch states must be released before the door movement completes.

# 4 Trace the middle module connection

Model labels: bay.module2, bay.module2.connectorright, bay.jumper2

Follow the training connection map from the middle module to its right connector and assigned jumper. Save this mapping.

Check: The learner identifies the intended connector and jumper together.

# 5 Resolve the connector discrepancy

Model labels: bay.module2.connectorright, bay.module3.connectorright, bay.jumper2

Present an instruction naming the lower module connector. Place a hold, record the conflicting identities and obtain the exercise reviewer correction.

Check: Selecting the lower-module connector after the discrepancy is a critical error.

# 6 Disconnect and stow the jumper

Model labels: bay.jumper2, bay.module2.connectorright, bay.harnessright

Disconnect the reviewed jumper route and place it in its authored stowed pose without pulling the adjacent harness.

Check: Record the correct endpoint and stow state before allowing module release.

# 7 Support the module

Model labels: bay.module2, bay.frame, bay.wall.bottom

Acknowledge the simulated module support and select its frame reference and lower clearance surface. Arm the removal sequence.

Check: Module withdrawal remains blocked until support is acknowledged.

# 8 Remove the middle module

Model labels: bay.module2, bay.frame, bay.module1, bay.module3

Translate the middle module along the approved demonstration path into its parked pose. Keep the upper and lower modules in place.

Check: Evaluate path order and final pose against the authored clearance envelope.

# 9 Refit the module

Model labels: bay.module2, bay.wall.bottom, bay.module2.connectorright

Return the module to the saved interface pose with its connector facing the intended side. Reject a reversed insertion.

Check: The seated pose and orientation must meet declared simulation tolerances.

# 10 Restore the connection

Model labels: bay.module2.connectorright, bay.jumper2, bay.harnessright

Reconnect the corrected jumper, restore its routed pose and compare the right harness with the baseline.

Check: Wrong endpoint, missing connection or displaced adjacent harness prevents completion.

# 11 Close the bay

Model labels: door.inner, door, door.latchupper, door.latchmiddle, door.latchlower

Check the door sweep against the restored harness, return the door to its closed pose and engage the three latches.

Check: Do not accept closure while any authored interference or open latch remains.

# 12 Release the maintenance handover

Model labels: bay.module2, bay.jumper2, door, stand

Compare final configuration with the baseline and submit the connector correction, ordered action record and required external test evidence.

Check: The reviewer sees the same model revision, restored module and resolved hold.

# Assessment and completion

Score component identification 20, ordered exchange 25, discrepancy resolution 20, restored configuration 20 and evidence 15. Unsupported withdrawal, wrong-connector continuation or missing isolation is a critical stop. A completed visual sequence does not establish electrical continuity or fastener torque.

# Authoring and headless handoff

Bind the real module, connector, jumper and door identifiers. Author support states, module parking poses, latch dependencies and door clearance explicitly. Do not infer a hinge axis, connector pinout or module electrical function. Use external records for programme-specific tests.

Reuse the exact GLB and component identifiers above. Verify the supported Genesis import and binding path; the current Creator scene driver can generate a different model. This document and its JSON describe authoring intent, not completed interactions. Preserve one model revision and record actual lesson, job and review identifiers during execution.